Info About API-5CT Coupling

- Couplings are short pipes with internal threads, used to connect two joints of tubing or casing.

- Role: Casing/Tubing Connection

- Application Scenarios: Down hole Drilling & Oil & Gas Production

- Standard：API 5CT & API 5B

- Production process: Saw Cutting, Machining Threading, Marking,

- Phosphate, Coating, Packing.

- Coupling Grade Colour Code: To distinguish different grades of steel.

- The meaning of grade designations:

“J/K/N/L/C/T/Q/P”---- Different properties

“55/80/90/95/125/110”---- Yield strength specification

- Common steel grades in the United States：J55,K55,L80,P110

- Package: By carton, wooden case, wooden pallet, or as customer's requirements.

- Data Sheet: Find out the dimensions, tolerances, and mass of couplings as specified in the API 5CT standard.

1.Casing Coupling

Size: 4-1/2” to 20”

Thread Type: STC - Short Round Thread

LTC - Long Round Thread

BTC - Buttress Thread

Grade: J55, K55, L80, N80-1, N80-Q, C90, T95, C110, P110, Q125

SCC: Special Clearance Coupling

Its internal space is slightly larger than that of standard couplings, allowing smooth connection even when the outer wall of the casing has a coating or slight deformation.

BBE: Bevel Both End

The two end edges of the coupling are beveled, which prevents casing scratches during installation and eases casing insertion.

Internal Chamfer Measurement         External Chamfer Measurement

Pictures Of Casing Coupling：

① Size:9-5/8”   Grade:K55   Thread:BTC

② Size:7-5/8”   Grade:L80   Thread:BTC

③ Size:8-5/8”  Grade: P110   Thread: BTC   SCC

2.Tubing Coupling

Size: 1.050” to 4-1/2”

Thread Type: EUE - External Upset End

NUE - Non-Upset End

Grade: J55,K55,L80,P110,13Cr

Pictures Of Tubing Coupling：

④ Size:2-7/8”   Grade:L80   Thread:EUE

⑤ Size:2-7/8”   Grade:P110   Thread:EUE    SCC